Main Specifications of the 2023 MG HS

## 📏 Dimensions & Weight

- Length × Width × Height: 4,574 × 1,876 × 1,664 mm

- Wheelbase: 2,720 mm

- Ground Clearance: 175 mm

- Kerb Weight: ≈1,550 kg (varies 1,516–1,775 kg depending on trim)

- Boot Space: 463 L (seats up); 1,375 L with seats folded

## 🚗 Powertrain Options

- 1.5 L Turbocharged Petrol (DCT)

  - Engine: 1,490–1,500 cc Turbo I4

  - Power: ~162 hp @ 5,500–5,600 rpm

  - Torque: 250–294 Nm @ 1,500–4,400 rpm

  - Transmission: 7-speed Dual-Clutch (DCT) or 6-speed manual in some markets

  - Drive: FWD

  - Top Speed: ~190 km/h

  - 0–100 km/h: ~9.6–9.9 s

- 2.0 L Turbo Petrol

  - Power: ~231 hp

  - Torque: 360 Nm

  - Transmission: 6-speed DCT

  - Drive: FWD (some AWD available)

  - Top Speed: ~210 km/h

- 1.5 L Turbo Plug-In Hybrid (PHEV)

  - Combined Power: ~258 hp

  - Combined Torque: 370 Nm

  - Battery: ~14.9–16.6 kWh

  - EV Range: ~32 km (WLTP)

  - Transmission: 10-speed EDU II auto

  - 0–100 km/h: ~6.9 s

## ⛽ Fuel & Efficiency

- 1.5 L Petrol: ~7.7 L/100 km combined (~30 US mpg)

- 2.0 L Petrol: ~9.5 L/100 km

- PHEV: ~1.8 L/100 km WLTP; EV range ~32 km

## 🛞 Chassis, Suspension & Brakes

- Front Suspension: MacPherson strut

- Rear Suspension: Multi-link

- Brakes: Front ventilated discs; rear solid discs with ABS/EBD

- Wheels/Tires: 18″ alloy wheels with 235/50 R18 tires

- Steering: Electric rack & pinion; turning radius ~5.9 m

## 🛋️ Interior & Tech

- Infotainment: 10.1″ touchscreen with MG Pilot safety suite (AEB, Lane Keep, ACC, Blind-Spot, 360-camera

- Climate: Dual-zone automatic AC; ambient lighting

- Comfort: Leather seats on higher trims; panoramic sunroof

- Safety: Up to 6 airbags, ESC, traction control; 5-star Euro NCAP rating

## 💸 Pakistan-Specific Info

- Prices: ~PKR 8.7–8.9 million (2023; Essence trim)

- Fuel Real-World:

  - City: ~7–10 km/L;

  - Highway: ~12–15 km/L depending on traffic and usage

- User Feedback (Reddit PK): Generally praised for drive and features, but flagged for DCT heat, fuel usage, and dependency on high-octane petrol

### ✅ Summary

The 2023 MG HS offers strong value with:

- A choice of efficient 1.5 L turbo petrol, more powerful 2.0 L petrol, or a compelling PHEV hybrid

- Spacious cabin (463 L boot) and family-friendly features

- Smooth ride and modern safety with advanced driver assistance

- PHEV presents ~32 km EV range, excellent fuel efficiency (1.8 L/100 km WLTP)

- In Pakistan, it’s well-equipped but can have higher fuel use and DCT quirks; premium fuel is essential